# Name & Student ID

Ben Openshaw

20376646

# Title

Robot Drawing Project Planning

# Outline of the Problem to be Solved

Explanation

The main goal of this project is to develop software to enable a robot to draw out specified text in a set writing area, transmitting commands from a file via a virtual RS232 serial port. Examples of this text can be found in the assignment brief.

Requirements

Git repository set up as a directory for developed code.

Frequent commits must be made for evidence of project progression.

Robot Drawing Process

The robot must first be switched on, set in the “up” position (S0). Pen speed should be set prior to loading any files (F1000), and serial connection to the RS232 serial device established. G-code M3 to turn on spindle, required for the arm to work.

Must start in position 0,0 (G1 0 0).

The font file SingleStrokeFont.txt should then be loaded for instruction on movement to the robot. This data must be stored as memory to ensure the system is not returning to the document each time it receives a command.

User input should set the required height between 4-10mm, as per brief.

TestData.txt, or the planned text file, should be loaded to provide instructions for the planned written text. As char function contains ASCII, no conversion function to ASCII would be required.

G-code commands will be given according to the previous order – look up font data, scale movement based on user input height, and ensure there are spaces between the characters (offset character position) to prevent the robot from writing immediately in the position it finishes the previous character. Convert these movements into G-code commands, and send them to the Arduino – await the response.

G0 X Y moves the pen to the position X,Y. G1 X Y draws a straight line from the last position to X,Y.

Larger spacing should also be included between words, and larger spaces for line breaks – e.g. move the pen down by a set height and return to a 0 horizontal position. The width of 100mm must be constantly checked to ensure the robot is not writing off the page – if it has been reached, it must start a new line returning to the 0 horizontal position.

Line breaks can also be applied where a word will not fit onto the rest of the available width space – this would prevent words from being split down the page, e.g. “HEL

LO”.

When finished, the robot must return to the position (0,0), top left, as per the brief. Ensure the pen returns to the “up” position.

# Key Data Items

| Name | Data type | Rationale |
| --- | --- | --- |
| asciiValue | int | Required for storing characters’ ASCII values |
| numberMovements | int | Number of steps taken for the drawing process |
| x[] | float array | Array of x position |
| y[] | float array | Array of y position |
| penState[] | int | Sets pen position – 0 for up, 1 for down (as pen should be up when starting) |
| currentX | float | Current x position as decimal |
| currentY | float | Current y position as decimal |
| fontSet[128] | FontChar | Stores the characters for lookup. ASCII range of 0-127 requires fontSet[128] to be fully covered. |
| textHold[] | char | Stores the current character being processed from the input text file. |
| textHeightmm | int | Sets text height in mm according to the user input |
| scaleFactor | float | Once height input is set, font units are scaled to a height between 4-10mm using scaleFactor |
| maxWidth | int | Allows maximum width of writing space of 100mm to be set |
| charSpacing | float | Sets horizontal spacing between characters at 2.0mm |
| wordSpacing | float | Sets greater spacing between complete words when program identifies ASCII 32 (space) |

Please note I was unsure if we are specifying .5 fonts, so have left textHeightmm as an integer value – this would change if specified.

# Function Declarations

Only include functions that you will develop.

Example (remove before submission)

int openText(char* filePath, char* textHold[])

- Parameters:

  - filePath – input: shows path to text file containing the required text

  - textHold – output: stores the characters read from the file

- Return: 1 if file can be opened and read successfully. If not, return 0.

int fetchFont(int asciiValue, FontChar* fontSet)

- Parameters:

  - asciiValue – input: ASCII code for the current character

  - fontSet – input: array of font data structures from 0-127 ASCII

- Return: 1 if font data could be successfully loaded, 0 if missing or invalid. Populate x[], y[], numberMovements, penState[] for current character.

int applyScaleFactor(float scaleFactor, float x[], float y[], int numberMovements)

- Parameters:

  - scaleFactor – input: factor for scaling font according to textHeightmm

  - x[] – input and output: sets movement array for character

  - y[] – input and output: sets movement array for character

  - numberMovements – input: number of steps required for character

- Return: 1 if arrays successfully scaled, 0 if invalid scaleFactor. Coordinate arrays updated for new scale.

int newPosition(float* currentX, float* currentY, float x[], float y[], int numberMovements)

- Parameters:

  - currentX – input and output: pen position updated after drawing

  - currentY – input and output: pen position updated after drawing

  - x[] – input: movement arrays required for character

  - y[] – input: movement arrays required for character

  - numberMovements – input: number of steps required for character

- Return: 1 if position updated successfully, 0 if failure due to e.g. invalid arrays. Pen position updated once complete.

int checkWidth(float current, int maxWidth)

- Parameters:

  - currentX – input: current position in the horizontal plane

  - maxWidth – input: max writing width set to 100mm

- Return: Return 1 only if line break is required (e.g. currentX => 100mm), else 0

int applyLineBreak(float* current, float* currentY)

- Parameters:

  - currentX – input and output: set new pen position to 0

  - currentY – input and output: set new pen position to 5mm further from 0

- Return: 1 if successful, 0 if failure due to e.g. invalid currentY, or max height reached (no max height given in brief, just adding as a potential addition). Sets pen position to 5mm lower and returns its horizontal position to the origin.

int generateGcode(int asciiValue, float x[], float y[], int penState[], int numberMovements)

- Parameters:

  - asciiValue – input: character’s ASCII value

  - x[] – input: coordinate array

  - y[] – input: coordinate array

  - penState[] – input: sets pen states, required for “up” and “down” settings

  - numberMovements – input: number of steps required

- Return: 1 if commands generated successfully, 0 if generation failure, e.g. empty arrays. G-code commands are produced for given character.

int applyCharacterSpacing(float* currentX, float charSpacing, float wordSpacing, int asciiValue)

- Parameters:

  - currentX – input and output: updated pen position for each character

  - charSpacing – input: spacing value set to 2mm

  - wordSpacing – input: if ASCII value = 32, increase currentX by +5

  - asciiValue – input: required for wordSpacing

- Return: 1 if successful, 0 if spacing failure, negative input etc. Pen position updated to leave a gap between characters and words.

int penStateOrigin(float* currentX, float* currentY, int* penState)

- Parameters:

  - currentX – input and output: reset to 0

  - currentY – input and output: reset to 0

  - penState – input and output: set to 0 (pen “up”)

- Return: 1 if successfully reset to origin, 0 if error present e.g. pen hasn’t moved. Pen moved back into original position, 0,0, in “up” state

int checkWordFit(char* textHold[], int startIndex, FontChar* fontSet, float charSpacing, float currentX, int maxWidth)

- Parameters:

  - textHold[] – input: text held in storage

  - startIndex – input: index of the current word’s first character

  - fontSet – input: font data for characters, defines required width

  - charSpacing – input: spacing between characters, 2mm

  - currentX – current X value, horizontal position

  - maxWidth – maximum width of page 100mm

- Return: 1 if word fits on line, 0 if word does not fit

# Testing Information

| Function | Test Case | Test Data | Expected Output |
| --- | --- | --- | --- |
| openText | Checking file is opened correctly, no errors in opening | File containing “HELLO WORLD” | textHold[] set to hold characters “H”, “E”, “L”, “L”, “O”, “ “, “W”, “O”, “R”, “L”, “D”. If error present, returns 0 and textHold[] is empty If TestData.txt is empty, program completes without drawing. |
| fetchFont | Check font data for character | asciiValue = 65 (A) | numberMovements, x[], y[], penState[] correctly populated |
| applyScaleFactor | Check scaling factor application | scaleFactor = 2 | x[] and y[] coordinates all doubled |
| newPosition | Check pen position updates correctly | currentX = 0, currentY = 0 | currentX and currentY update to final position |
| checkWidth | Check line break is triggered only when needed | currentX >= maxwidth maxwidth = 100mm | Return 1 if line break required Else return 0 |
| applyLineBreak | Check line break application | currentX = 110 currentY = 10 | currentX becomes 0, currentY increases by 5 |
| generateGcode | Check G-code generation | asciiValue = 65 (A) with date filled | Produced G-code commands set pen up and down positions, correct X and Y movements for A. |
| applyCharacterSpacing | Check correct spacing between characters and words | Text “HELLO WORLD” | Correct spacing of 2mm between characters, and 5mm between “HELLO” and “WORLD”. |
| penStateOrigin | Check that the pen returns to the origin point at completion | currentX = 10, currentY=10, penState=1. Test with empty .txt file. | Pen should return to currentX = 0, currentY = 0, penState = 0. |
| checkWordFit | Check if word fits on the current line | “HELLO” at currentX = 90, maxWidth = 100 | Return 0, line break must be applied |
| checkWordFit | “ | “HELLO” at currentX = 10, maxWidth = 100 | Return 1, no line break needed |
| main() | Check SingleStrokeFont.txt opens correctly | No SingleStrokeFont.txt present | Program error, terminates. |
| main() | Check correct beginning-to-end function | File “HELLO WORLD”, set textHeightmm = 6 | End product shows HELLO WORLD with correct spacing between letters, fitting within 100mm width, line breaks where required, returns to 0,0 with pen in “up” position. |

# Flowchart(s)

May be included as separate pdf

# AI Statement

In writing this statement, I confirm that no AI assistance was used to put this work together. All work presented was made using the help only of lecture notes and exercises for the help to understand the brief and action on its requirements.